2.1 Histopathology Paper

Class March 2022

- Fixatives that are employed when the preservation of intracellular structures or inclusion is of first importance are termed as;

- Special

- Microanatomical

- Cytological

- Simple

- Kulchitzky cells are also termed as ;

- Copper

- Lipofuchsin granules

- Argentaffin granules

- Chromaffin granules

- Gomori lead  method is used for the demonstration of:

- Dopa oxidase

- Lipase

- Alcaline phosphates

- Acid phosphates

- Method of decalcification which acts by simply dissolving calcium ion in the tissue is

- Mineral acid

- Ion exchange resin

- Chelating agent

- Electrolysis

- Benzine compounds which contains chromophore are termed as:

- Auxochrome

- Chromogen

- Chromophore

- Dyes

- A method of tissue biopsy collection where the whole organ or tissue is removed is

- Incision biopsy

- Cone biopsy

- Excision  biopsy

- Core biopsy

- Endpoint of dehydration can be confirmed using

- Anhydrous copper sulphate

- Picric acid

- Mercuric chloride

- Electrolysis

- The following clearing agent has toxic effects on exposure to high concentration of fumes for a long period

- Toluene

- Xyline

- Chloroform

- Benzene

- Celloidine is used in the following concentration

- 3%,6%,8%

- 2%,4%,8%

- 5%,10%,20%

- 15%,20%,30%

- Method of embedding that seeks to achieve the advantage of both paraffin wax and celloidin embedding is

- Paraffin wax

- Low viscosity nitrocellulose

- Pertefis double embedding

- Gelatin

- Scores seen in tissue sections may be due to

- Soft paraffin wax

- Loose knife

- Improper fixation

- Knife tilt too great

- Metachromatic dyes includes

- Thionine

- Haematoxylin

- Eosin

- Carbol fuchsin

- Abrassive (s) used during microtome knife sharpening includes

- Mercuric chloride

- Starch paste

- Alluninium oxide

- Osmium tetraoxide

- Trimming of tissue block is to

- Expose the surface of tissue

- Avoid hardening of tissue

- Evade damage to the knife

- Obtain serial sections

- Bile pigments seen in tissue sections are also known as

- Hemosiderin

- Hematoidin

- Haemozoin

- Melanin

- Accentuators used in conjunction with histological dyes include

- Ammonium alum

- Ferric chloride

- Iodine

- Potassium hydroxide

- Washing out is mainly done on tissues

- Fixed in 10% formalin

- Cleared in xylene

- Fixed in potassium dichromate containing fixatives

- Formaldehyde

- Natural dye recommended for the demonstration of Australian antigen and elastic fibres is:

- Orcein

- Haematoxylin

- Brazilin

- Verhoeffs

- Synthetic dyes are organic compounds consisting of :

- Alum

- Benzene ring

- Oxidants

- Mordants

- Water soluble waxes :

- Penetrates fatty tissues rapidly

- Causes little tissue shrinkage

- Sections are easy to float

- Blocks must be kept in hygroscopic containers

Section B Short Answer questions

- Outline factors that determine the fixative and sectioning method to be used during tissue processing.                                                ( 5 Marks)

- Explain how to perform endpoint of decalcification using chemical method (5marks)

- Explain the word impregnation and outline its significant  in tissue processing                                                   ( 5marks)

- Outline five tissues that benefit from vacuum embedding                           (5marks)

- Describe haematoxylin as a natural dye                                                       ( 5marks)

- List five mechanisms/methods of staining                                                    ( 5marks)

- Outline autogenous pigments                                                                        ( 5 marks)

- Explain mounting and outline the aim of mountants in tissue preparation    ( 5 marks)

Section C Long essay questions

- Dazy a histopathologyst at the KNH hospital had several tissues to process for microscopy,after microtomy she had to mount the tissues on the slide for staining .Explain the procedure that Daizy would use to attach the tissue to the slide.   (20 marks)

- Discuss theories of staining                                                                               ( 20 marks)

Making key

- C

- C

- D

- A

- B

- C

- A

- A

- B

- C

- D

- A

- C

- A

- B

- D

- C

- A

- B

- B

- Factors that determines fixative and sectioning method to be used

- Urgency of examination

- Structure or inclusion to be demonstrated

- The material to be sectioned

- Staining procedure to be used/employed

- Whether or not serial sections are required

- End point of decalcification using chemical method

Put 5mls of used decalcifying fluid in test-tube and add apiece of red litmus paper

Add strong ammonia drop by drop until the red litmus paper turns blue to show that the decalcifying fluid is alkaline.

Turbidity at this stage shows that the fluid contains plenty of calcium

If the solution remains clear add 1ml of 5% ammonium oxalate and let it stand for 30minutes

Turbidity at this stage shoes that there is little calcium in the decalcifying solution and no turbidity shows that decalcification is complete.

- Impregnation and significant in tissue processing

- Is the process of saturating the tissue with media to be used for embedding.

- Significance

- To saturate the tissue with the medium to be used for embedding

- To remove clearing agent from the tissue in order to allow complete infiltration by paraffin wax so that it forms a solid block from which the tissue section can be cut

- To make tissue completely permeable to paraffin wax

- Tissues that benefit from vacuum embedding

Tough fibrous tissues

Dense tissues e.g the brain

Hard tissues e.g the bones

Air containing tissues e.g the lungs

Urgent biopsy

Tissues with lots of fats e.g the breast

Blood containing tissues e.g the spleen and liver

Poorly penetrated tissue eg the skin

Tissues made of delicate structures and mixed consistencies   e.g the eye

Lymphnodes

- Haematoxylin

It is anatural dye

Extracted from a small tree called campechianum

Extracted from its pith by ether

Used in conjunction with the mordant

It is ripened or oxidesed by haematin and has poor staining properties

- Mechanisms of staining

Non vital staining

Metallic impregnation

Histochemical methods

Vital staining

Metachromatic staining

Elective solubility staining

Staining by natural or synthetic dyes

- Autogenous pigments

Melanin, argentaffin granules, chromaffin granules, lipofuschin and copper.

- Mountants

Is a process of covering the section with suitable medium and a coverslip for safe keeping and examining without the fear of damages and scratching the sections.              ( 2 marks)

Aim.

To prevent the section from being scratched.                                                         ( 1mark)

To preserve the stain in the tissue.                                                                          ( 1mark)

To protect the stain section from termite                                                                  ( 1mark)

Long  answer questions 60 marks

- Procedure for attaching the tissue to the slide

- After microtome, take a clean grease free slide

- By the use of a camel hair brush, pick the tissue from a microtome stage

- Place the tissue on top of the slide

- Place 20% of alcohol between the slide and the tissue.

- This is to remove creases and folds

- Release the tissue to the water bath

- The water bath controlled at a temperature of 2-3⁰C below the melting point of wax

- Using a pin or needle, divide the tissue into the sections(small)

- Take a clean grease free slide

- Apply an adhesive on the slide

- Insert the slide into the water bath next to the tissue at an angle of 45⁰C fish the tissue from the water bath

- Orientate the tissue on top of the slide

- Label the slide with pts no.

- Take to the hot air oven to remove wax

- Theories of staining

There are two theories of staining;

Physical  theory.

chemical theory.

PHYSICAL THEORY.

- Is governed by four factors namely;

- Solubility absorption

- Adsorption

- Density

- Permeability

Solubility Absorption

This theory states that dyes are soluble in fats/tissue than in solvent used such as 70% ethanol. The dye colors the tissue such dyes that dissolve in tissue are called lysochrome.

Almost all lysochromes are lipid soluble and are used in histology for the demonstration of lipids.

Examples of these are;      Sudan III

Sudan black B

Oil red

- Adsorption

Also called electrical theory.

This is the property by which small object especially in a fine state get attracted by large substances from surrounding media and get attracted on to their free surfaces.

Example is metachromatic staining.

- Density

Is where more proteneous tissue constituents take up the dye more intensely than other structures.

Dense tissue substances that have large protein chain unit per unit volume holds a large amount of dye and will remain coloured long after less dense tissue have been discolored by differentiation.

- Permeability

Is the proximity of space.

Highly permeable tissue will be quickly stained and differentiated while less permeable will require longer time to acquire stain and more resistant to fibrous tissues take in more stain than the less fibrous tissue.