Basis of Measurements

Measurement is the basic concept in the study of Mathematics and Science. Measurement quantifies the characteristics of an object or event, which we can compare with other things or events. Measurement is the most commonly used word, whenever we deal with the division of a quantity

Types of Measurement

There are some basic types of measurement variables and their SI units that we deal with in several areas of mathematics. They are:

Length - meter (m)

Time - second (s)

Amount of substance - mole (mole)

Electric current - ampere (A)

Temperature - kelvin (K)

Luminous intensity - candela (cd)

Mass - kilogram (kg)

The data is classified into majorly four categories:

Nominal data

Ordinal data

Discrete data

Continuous data

Qualitative or Categorical Data

Qualitative data, also known as the categorical data, describes the data that fits into the categories. Qualitative data are not numerical. The categorical information involves categorical variables that describe the features such as a person’s gender, home town etc. Categorical measures are defined in terms of natural language specifications, but not in terms of numbers.

Nominal Data

Nominal data is one of the types of qualitative information which helps to label the variables without providing the numerical value. Nominal data is also called the nominal scale. It cannot be ordered and measured. But sometimes, the data can be qualitative and quantitative. Examples of nominal data are letters, symbols, words, gender etc.

The nominal data are examined using the grouping method. In this method, the data are grouped into categories, and then the frequency or the percentage of the data can be calculated. These data are visually represented using the pie charts.

Ordinal Data

Ordinal data/variable is a type of data that follows a natural order. The significant feature of the nominal data is that the difference between the data values is not determined. This variable is mostly found in surveys, finance, economics, questionnaires, and so on.

The ordinal data is commonly represented using a bar chart. These data are investigated and interpreted through many visualisation tools. The information may be expressed using tables in which each row in the table shows the distinct category.

Quantitative or Numerical Data

Quantitative data is also known as numerical data which represents the numerical value (i.e., how much, how often, how many). Numerical data gives information about the quantities of a specific thing. Some examples of numerical data are height, length, size, weight, and so on. The quantitative data can be classified into two different types based on the data sets. The two different classifications of numerical data are discrete data and continuous data.

Discrete Data

Discrete data can take only discrete values. Discrete information contains only a finite number of possible values. Those values cannot be subdivided meaningfully. Here, things can be counted in whole numbers.

Example: Number of students in the class

Continuous Data

Continuous data is data that can be calculated. It has an infinite number of probable values that can be selected within a given specific range.

Example: Temperature range

2. Measurement standards

Measurement standards are those devices, artifacts, procedures, instruments, systems, protocols, or processes which are used to define (or to realize) measurement units and on which all lower level (less accurate) measurements depend. A measurement standard can also be said to store, embody, or otherwise provide a physical quantity which serves as the basis for the measurement of the quantity.

A measurement standard is defined as the physical embodiment of a measurement unit, by which its assigned value is defined, and to which it can be compared for calibration purposes

Standards

they are of several kinds.

1. Measurement standards

2. Standards of practice or protocol standards-

3. Legal Metrology

4. Forensic Metrology

5. Standard Reference Materials

Types of standards

There are two types of standards namely

(i) basic or fundamental standards, and

(ii) derived standards.

Basic or fundamental standards – In the SI system, there are seven basic measurement units from which all other units are derived. All of the units except one are defined in terms of their unitary value. The one exception is the unit of mass. It is defined as 1000 grams (g) or 1 kilogram (kg). It is also unique in that it is the only unit currently based on an artifact. All the standards of mass are based on one particular platinum / iridium cylinder kept at the BIPM in France. If that International Prototype Kilogram is to change, then all other mass standards throughout the world becomes wrong.

The seven basic units are listed in Tab 1.

| Tab 1 SI Base Units |  |  |  |
| --- | --- | --- | --- |
| Sl. No. | Base quantity | Name of Base Unit | Symbol |
| 1 | Length | meter | m |
| 2 | Mass | kilogram | kg |
| 3 | Time | second | s |
| 4 | Electric current | ampere | A |
| 5 | Thermodynamic temperature | kelvin | K |
| 6 | Amount of substance | mole | mol |
| 7 | Luminous intensity | candela | cd |

The International definitions of the SI Base Units are given below.

Unit of length (meter) – The meter is the length of the path traveled by light in vacuum during a time interval of 1/299,792,458 of a second.

Unit of mass (kilogram) – The kilogram is the unit of mass which is equal to the mass of the international prototype of the kilogram.

Unit of time (second) – The second is the duration of 9,192,631,770 periods of the radiation corresponding to the transition between the two hyperfine levels of the ground state of the cesium-133 atom.

Unit of electric current (ampere) – The ampere is that constant current which, if maintained in two straight parallel conductors of infinite length, of negligible cross section, and placed 1 meter apart in vacuum, would produce between these conductors a force equal to 0.0000002 newtons per meter of length.

Unit of thermodynamic temperature (kelvin) – The kelvin, unit of thermodynamic temperature, is the fraction 1/273.16 of the thermodynamic temperature of the triple point of water. The unit kelvin and its symbol K is also used to express an interval or a difference in temperature..

Unit of amount of substance (mole) – The mole is the amount of substance of a system which contains as many elementary entities as there are atoms in 0.012 kilogram of carbon-12.

Unit of luminous intensity (candela) – The candela is the luminous intensity, in a given direction, of a source which emits monochromatic radiation of frequency 540 x (10 to the power 12) hertz and which has a radiant intensity in that direction of (1/683) watt per steradian.

Derived Standards

All of the other units are derived from the seven basic units described in Tab 1. Measurement standards are devices which represent the SI standard unit in a measurement. (As an example, one can use a zener diode together with a reference amplifier and a power source to supply a known voltage to calibrate a digital voltmeter.

ACCURRACY AND PRECISION.

The accuracy of a measurement is the relative exemption from errors. That is, accuracy is the measure of how close the measured value is to the actual value of the quantity.

For every instrument, there is a minimum value that can be measured accurately. This is called the least count of that instrument. It is 0.1 cm for an ordinary scale, 0.01 cm for an ordinary Vernier caliper and 0.001 cm for an ordinary screw gauge.

Precision describes the limit or resolution of the quantity measured. For example, consider an iron rod of length 12 cm. The scale 1 measures it to be 11.9 cm and scale 2 measures it to be 12.426 cm. Here scale 1 is more accurate but scale 2 is more precise.

Variation and error

Measurement system variation is the variation that occurs when you measure something. Specifically, measurement system variation is the sum of variation from repeatability and reproducibility.

Any component of a measurement system, such as a gage, a procedure, and software, can be a source of variation.

Example of measurement system variation

Operators at a manufacturing company use calipers to measure the diameters of shafts. A known standard shaft has a diameter of 5 cm. When measured multiple times, the measurements are: 5.01 cm, 4.99 cm, 4.97 cm, 5.03 cm, 5.01 cm. The variation in the measurements is due to measurement system variation.

Types of Errors

There are three types of errors that are classified based on the source they arise from; They are:

Gross Errors

Random Errors

Systematic Errors

Gross Errors

This category basically takes into account human oversight and other mistakes while reading, recording, and readings. The most common human error in measurement falls under this category of measurement errors. For example, the person taking the reading from the meter of the instrument may read 23 as 28. Gross errors can be avoided by using two suitable measures, and they are written below:

Proper care should be taken in reading, recording the data. Also, the calculation of error should be done accurately.

By increasing the number of experimenters, we can reduce the gross errors. If each experimenter takes different readings at different points, then by taking the average of more readings, we can reduce the gross errors

Random Errors

The random errors are those errors, which occur irregularly and hence are random. These can arise due to random and unpredictable fluctuations in experimental conditions (Example: unpredictable fluctuations in temperature, voltage supply, mechanical vibrations of experimental set-ups, etc, errors by the observer taking readings, etc. For example, when the same person repeats the same observation, he may likely get different readings every time.

This article explored the various types of errors in the measurements we make. These errors are everywhere in every measurement we make. To find more articles, visit BYJU’S. Join us and fall in love with learning.

Systematic Errors:

Systematic errors can be better understood if we divide them into subgroups; They are:

Environmental Errors

Observational Errors

Instrumental Errors

Environmental Errors: This type of error arises in the measurement due to the effect of the external conditions on the measurement. The external condition includes temperature, pressure, and humidity and can also include an external magnetic field. If you measure your temperature under the armpits and during the measurement, if the electricity goes out and the room gets hot, it will affect your body temperature, affecting the reading.

Observational Errors: These are the errors that arise due to an individual’s bias, lack of proper setting of the apparatus, or an individual’s carelessness in taking observations. The measurement errors also include wrong readings due to Parallax errors.

Instrumental Errors: These errors arise due to faulty construction and calibration of the measuring instruments. Such errors arise due to the hysteresis of the equipment or due to friction. Lots of the time, the equipment being used is faulty due to misuse or neglect, which changes the reading of the equipment. The zero error is a very common type of error. This error is common in devices like Vernier callipers and screw gauges. The zero error can be either positive or negative. Sometimes the scale readings are worn off, which can also lead to a bad reading.

Instrumental error takes place due to :

An inherent constraint of devices

Misuse of Apparatus

Effect of Loading

Errors Calculation

Different measures of errors include:

Absolute Error

The difference between the measured value of a quantity and its actual value gives the absolute error. It is the variation between the actual values and measured values. It is given by

Absolute error = |VA-VE|

Percent Error

It is another way of expressing the error in measurement. This calculation allows us to gauge how accurate a measured value is with respect to the true value. Per cent error is given by the formula

Percentage error (%) = (VA-VE) / VE) x 100

Relative Error

The ratio of the absolute error to the accepted measurement gives the relative error. The relative error is given by the formula:

Relative Error = Absolute error / Actual value

How To Reduce Errors In Measurement

Keeping an eye on the procedure and following the below listed points can help to reduce the error.

Make sure the formulas used for measurement are correct.

Cross check the measured value of a quantity for improved accuracy.

Use the instrument that has the highest precision.

It is suggested to pilot test measuring instruments for better accuracy.

Use multiple measures for the same construct.

Note the measurements under controlled conditions.

Frequently Asked Questions – FAQs

What is meant by measurement error?

The difference between a measured quantity and its true value gives measurement error.

What are the types of errors?

The following are the types of errors:

Gross Errors

Random Errors

Systematic Errors

The error seen due to the effect of the external conditions on the measurement is known as?

It is known as the environmental error.

Define absolute error?

Absolute error is the variation between the actual values and measured values. It is given by
Absolute error = |VA-VE|

Combination of errors

When a quantity is determined by combining several measurements, the errors in the different measurements will combine in some way or other.

Error in the sum of the quantities

Let two quantities, A and B have measured values A ± ∆ A and B ± ∆ B respectively.
∆A and  ∆B are the absolute errors in their measurements.

To find the error ∆Z that may occur in the sum Z = A + B, consider
Z ± ∆Z = (A ± ∆A) + (B ± ∆B)
            = A + B ± ∆A ± ∆B

The maximum possible error in the value of Z is given by ∆Z = ∆A + ∆B.

Thus, when two quantities are added, the absolute error in the result is the sum of the absolute errors in the measured quantities.

Error in the difference of the quantities

Let two quantities, A and B have measured values A ± ∆ A and B ± ∆ B respectively.
∆A and  ∆B are the absolute errors in their measurements.

To find the error ∆Z that may occur in the difference Z = A – B, consider
Z ± ∆Z = (A ± ∆A) – (B ± ∆B)
           = A – B ± ∆A ± ∆B

Similarly, the maximum possible error in the value of Z is given by ∆Z = ∆A + ∆B.

Thus, when two quantities are subtracted, the absolute error in the result is the sum of the absolute errors in the measured quantities.

Error in the product of the quantities

Let two quantities, A and B have measured values A ± ∆ A and B ± ∆ B respectively.
∆A and  ∆B are the absolute errors in their measurements.

To find the error ∆Z that may occur in the product Z = AB, consider
Z ± ∆Z = (A ± ∆A)(B ± ∆B)
           = AB  ± A∆B ± B∆A ± ∆A∆B

Dividing L.H.S by Z and R.H.S by AB we get,

Since ∆A and ∆B are small, their products  is very small and can be neglected. Hence, the maximum fractional error in Z is given by

Thus, when two quantities are multiplied, the fractional error in the result is the sum of the fractional errors in the measured quantities.

Error in the quotient of the quantities

Let two quantities, A and B have measured values A ± ∆ A and B ± ∆ B respectively.
∆A and  ∆B are the absolute errors in their measurements.

To find the error ∆Z that may occur in the quotient , consider

Similarly, on solving, we get the maximum fractional error in Z as

Thus, when two quantities are divided, the fractional error in the result is the sum of the fractional errors in the measured quantities.

Error when a quantity is raised to a power

The error ∆Z that may occur when a quantity is raised to its nth power is n times the fractional error in the quantity itself ie., if Z = An ,
   ——————- eqn (1)

By this equation  it is clear that the maximum percentage error in Z is the sum of the maximum percentage error in A and maximum percentage error in B.

Similarly from eqn (1), the maximum percentage error in Z is given by

If , then the maximum % error in Z is given by

For example:- The volume V of a cube of side L is given by
V = L3 = L.L.L
Thus as before, 
or, the maximum fractional error

Error calculation solved problems

1. A physical quantity p is related to four observations a, b, c and d as follows  . The percentage error in the measurements in a, b, c and d are 2 %, 3 %, 1 % and 4 % respectively. Calculate the percentage error in p?

Ans:

ie, % error in p = 2×(% error in a)+3×(% error in b)+(% error in c)+ ½ ×(% error in d).
= 2 x 2 % + 3 x 3 % + 1 % + ½ x 4 %
= 16 %

2. Period of oscillations of a simple pendulum is measured in an experiment to be 2.01 s, 2.03 s, 1.99 s, 1.98 s, 2.05 s and 2.04 s. Calculate (1) Mean time period of the pendulum, (2) Absolute error in each measurements (3) Average absolute error (4) Relative error and (5) Percentage error.

Ans:

Let T1 = 2.01 s, T2 = 2.03 s, T3 = 1.99 s, T4 = 1.98 s, T5 = 2.05 s, T6 = 2.04 s

(1) Mean time period of the pendulum is

Since there are only three significant figures, it is proper to have only 3 significant figures in the mean also
ie, Tmean = 2.02 s

(2) Absolute errors in each measurement.

| Absolute errors = Difference between Tmean and measured value | Magnitude of errors |
| --- | --- |
| ∆T1 = Tmean – T1 = 2.02 – 2.01 = 0.01 s | 0.01 s |
| ∆T2 = Tmean – T2 = 2.02 – 2.03 = -0.01 s | 0.01 s |
| ∆T3 = Tmean – T3 = 2.02 – 1.99 = 0.03 s | 0.03 s |
| ∆T4 = Tmean – T4 = 2.02 – 1.98 = 0.04 s | 0.04 s |
| ∆T5 = Tmean – T5 = 2.02 – 2.05 = -0.03 s | 0.03 s |
| ∆T6 = Tmean – T6 = 2.02 – 2.04 = -0.02 s | 0.02 s |

(3) Average absolute error.

Average absolute error = arithmetic mean of the magnitude of the errors

(4) Relative error.

(5) Percentage error.

3. Calculate the maximum percentage error in P if  . Given that r = (0.32 ± 0.03) cm; X = (19 ± 1); L = (72 ± 0.2 )cm; π is a constant.

Ans:

Given 

Maximum percentage of error in p = 0.244 × 100 = 24.4 %

4. Two resistances (80 ± 3)Ω and  (130 ± 4)Ω are connected in series. Calculate the effective resistance with error limit and percentage error?

Ans:

Let R1 = 80 Ω, ∆R1 = 3 Ω, R2 = 130 Ω, ∆R2 = 4 Ω

Effective resistance of this series connection

R = R1 + R2 = 80 + 130 = 210 Ω
∆R = ∆R1 + ∆R2 = 3 + 4 = 7 Ω

∴ Effective resistances with error limit = (210 ± 7)Ω

5. The time period of oscillation of a simple pendulum is . The length of the pendulum is measured with a scale of least count 1 mm is 60 cm. If the time for 20 oscillations is measured with a stop watch of resolution 0.1 s is 50 s, what is the percentage error in the determination of g?

Ans:

Given ∆l = 1 mm = 0.1 cm
l = 60 cm
Time for 20 oscillations = 50 s
∆T = 0.1 s

A length was calculated to be 10.1 feet, but the absolute length was 10.5 feet. Calculate the absolute error.

We know that, Absolute error = |VA-VE|
Absolute error = 10.5-10.1 = 0.4 feet

CATEGORIES of MEASURENT

This categories includes;

1. Direct method;                                                                                                          2. Indirect

a)Deflection

b)Comparison:-   -null

-deferential

1. Direct

This is a simple method of measurement, in which the value of quantity to be measured is obtained directly without any kind of calculation.

Example use of scales, Vernier caliper and etc.

This method is not accurate because it depends on human insensitiveness in making judgement.

a) Comparison method

In this unknown values are compared to known values of some quantities or the quantity of functionally related to it. E.g. temperature by mercury thermometer

i) Null type of Comparison

In this method, the quantity to be measured is compared to a known quantity and the difference between these two is zero. E.g. in balance machine measuring mass’

ii) Differential method

Measuring pressure using pressure gauge where pressure is measured through two different liquids of different densities.

b) Deflection

The value of unknown quantity is directly indicated by deflected of the pointer on a calibrated scale

2. Indirect

Indirect method the value of quantity to be measured is obtained by measuring other quantities which are functionally related to the required value

E.g. angle measurement by sine bar, measurement of screw pitch diameter by three wire method etc.

FACORS THAT INFLUENCE THE RESULT OF MEASURENT

Equipment

The equipment selected to perform tests and calibrations is important. It can severely impact the uncertainty in measurement results. Hence, the reason many laboratories and professionals have been known to be very selective when purchasing equipment., reproducibility, stability, etc.). Therefore, it is important to consider what equipment will be used to perform measurements.

02 | Unit Under Test

When making measurements, it is important to consider the unit under test. Typically, the unit under test, or UUT, is often overlooked as having an influence on uncertainty. However, the unit under test can actually contribute a lot of uncertainty to the measurement result. All of these factors contribute to the uncertainty in the measurement result. Therefore, it is important to consider the resolution and the measurement repeatability of the unit under test.

03 | Operator

The operator who performs tests and calibrations has a major influence on the uncertainty associated with a measurement result. Their ability to facilitate the measurement process and perform quality work has a direct impact on the measurement result. Factors such as education, training, expertise, and technique can all influence uncertainty. Therefore, it is important conduct reproducibility experiments between operators to quantify the uncertainty (i.e. variability) in the measurement results as a result of the operator.

04 | Environment

The environment where tests and calibrations are performed can have an influence on uncertainty in measurement results. Variables such as temperature, humidity, pressure, gravity, elevation, vibration, stress, strain, lighting, etc. can impact the measurement result. Some tests and calibrations are more sensitive to certain environmental factors than others. Therefore, it is important to have expertise related to the specific measurement discipline or system which is being evaluated. An oversight on the influence of the environment not only impacts uncertainty, but may result in additional measurement error.

05 | Method

Test and calibration methods prescribe how a measurement process shall be performed by the operator. However, similar measurement results can be accomplished with the use of different methods (i.e. equifinality).  The variability in the measurement result that is attributed to the method is what influences uncertainty. When a laboratory utilizes several methods to perform similar measurements, it is important to consider how the method will affect the uncertainty of the measurement result.

06 | Calibration Service Provider

When having equipment serviced or calibrated, it is often wise to consider who will be providing service. Service providers have a direct impact and influence on the uncertainty in measurement results via traceability. When the service provider reports an associated estimate of uncertainty on the calibration report, the value is considered the traceable uncertainty or the reference standard uncertainty. It is impossible to legitimately claim an estimate of uncertainty for a measurement result that is less than the traceable uncertainty.