SECTION ADHESIVES, MOUNTING AND RINGING MEDIA.

SECTION ADHESIVES

These are syrupy fluids or pastes which are used to produce a firm bounding between un stained  tissue and a slide. They are commonly used in staining techniques in which reagents such as ammonia are used, in particular silver techniques and in staining processes where the tissue section is required to be immersed in the staining solution for long periods.

In both circumstances above, there is a tendency of the sections to de touch from the slide and therefore adhesives are used for firm bonding. They are not necessary in the routine H and E staining methods, which require only sufficient or thorough drying of sections.

They may be applied on the glass slide or may be put in the warm water bath. Examples of adhesives range from quice jelly to garlic water and include sugar, gum, starch, gelatin or human or bovine or egg albumin.

Other adhesives that can be used are cellulose, resin and sodium silicate. The commonly used adhesives are:

- Mayer’s egg albumin

- Starch paste

- Gelatin

PREPARATION:

- MAYER’S   EGG ALBUMIN.

- Mix well equal parts of egg white, distilled water and glycerin. Filter in a coarse filter using negative pressure .Add a thymol crystal as a preservative to prevent growth of moulds and bacteria in the adhesive.

- STARCH PASTE.

-Starch Paste 1 g

-10 ml cold distilled water

-mix

-pour the mixture into 20 ml of hot distilled water

-Add 20 drops of 1 N HCl, continue boiling for 4-5 minutes

-cool then add thymol crystals.

3.  GELATINE

Stock solution

-1% gelatin

- 1% potassium dichromate NB: Working solution:   Equal volumes of above.

HOW ADHESIVES ARE APPLIED:

-Use a glass rod, applicator stick or pipette to place a drop on a slide

-smear using a ball of the finger with gauze.

- Use the slide immediately to pick the section from the water bath

- dry in an incubator and store in a dust free place.

DISADVANTANGES OF ADHESIVES.

- If it is applied heavily on the slide and the slide stained, the albumin also stains and gives dirty background.

- Some thymol resistant organisms and moulds grow and the medium (adhesive) may become a source of fungal infections attacking finger nails and also a source of contamination to the section preparations.

NB: Starch paste is the best .It provides a clear firm adhesion to the section.

MOUNTING MEDIA

Mounting is a process of covering the sections with a suitable medium and cover slip. This medium is a syrupy fluid used between a stained section and a cover slip so that on drying, it becomes hard and firm preventing the movement of the cover slip. It therefore preserves and protects a stained tissue section. The choice of mounting medium depends on the staining procedure used.

Most of the mount ants become hard on setting and have a refractive index (R.I) as near as possible to that of glass between ( 1.48-1.52 ).Mounting media commonly used  are divided into two groups:-

  - Aqueous mountants

  - Resinous mountants

-natural

-synthetic

AQUEOUS MOUNTANTS (water based)

Most of the mount ants are made of water therefore stained tissue sections must be mounted straight from water. They should not be treated with alcohol or xylene after staining. Generally aqueous mount ants are temporary but still can make permanent preparations. They are mostly used to mount frozen sections stained for demonstration of fats or sections for metachromacy.

The general formula for the mount ants include:-

- A solidifying agent e.g.  gelatin

- Glycerin- to prevent drying and cracking.

- Sugars- for increasing refractive index

- Preservative- e.g. phenol, thymol, etc.

  - GLYCERINE JELLY ( R.I-1.47 )

Gelatin 		-10g

Glycerin	-70mls

Distilled water	-60 mls

Phenol		-0.25 g

PREPARATION

Dissolve gelatin in water and incubate or heat at 60 oc to enhance dissolving, add glycerin and phenol crystals. Mix well and store at 4 0c. For use, melt at 60 o c.

  - FARRANT’S MEDIA

Gum Arabica	-50 g

Glycerin	-50mls

Distilled water	-50 mls

Arsenic acid	- 1.0 gm

Preparation is as for glycerin jelly.

3.APATHY’S MEDIA R.I 1.52 (commonly used)

Gum Arabic		- 50gm

Cane sugar		-50 gm

Distilled water	-50 mls

Thymol 		-0.5 g

NB: Because of its high R.I, it is the most commonly used aqueous media.

PREPARATION

Dissolve ingredients in distilled water and store in screw capped bottles at 4 o c.

RESINOUS MOUNTANTS (NATURAL, SYNTHETIC)

These are composed of resins which can be natural or synthetic. They are designed for permanent preparations. They mount sections which have been stained, dehydrated and cleared e.g in cytology and histology.

- Resinous -Natural resins

Mostly from trees, they include Canada balsam (R.I 1.52), Dammar balsam, Euparol (R.I-1.48) and Colophonium resin etc. Canada balsam is obtained from a Canadian fir tree called Abies Balsamea.

It is prepared by dissolving 50-60 % by weight in xylene and adding calcium carbonate and filter to give a neutral balsam, while if salicyclic acid is substituted for calcium carbonate, we get an acidic balsam. Dammar balsam comes from a tree an Indian tree called Sherea Wiesueri.

Euparol : Refractive index 1.48. It is prepared by dissolving sandarac resin in a mixture of eucalyptus, paraldehyde and camsal.  It is miscible with xylene and alcohol.

- Resinous-Synthetic resins

These are available commercially .They include D.P.X, B.P.S. and Lendrum. They are colourless mount ants available commercially and have come to replace Canada balsam. They are prepared by dissolving the following:

- Distrene  ”80’-’( polysterene )- 10g

- Plasticizer - Dibutylphthatate -5ml

- Xylene- an aromatic hydro-carbon solvent.-35 ml

Preparation

- Combine the dibutylphhthalate with the xylene.

- Mix and dissolve the distrene ‘80’

- Record and label.

D.P.X is the most commonly used synthetic mountant.

CRITERIA OF GOOD MOUNTING MEDIA

- They should not crack on setting

- They should set quickly

- They should not react with the stains used.

- They should be clear

- They should be free from dirt and dust

- They should be air bubble free

- They should have a syrupy consistency to facilitate uniform application of drops.

- They should not retract on setting

- They should not cause sections to fade

- They should be mould free,  if aqueous

- They should be approximately the same R.I (refractive index) as the structures being demonstrated.

RINGING MEDIA

Ringing- is a procedure done on a stained and mounted section by sealing the margins of the cover slip  using molten paraffin wax or nail vanish.

Purpose

- avoid air in to cover slipped preparation.

- Evaporation of the mountant . Ringing media harden fairly rapidly trapping the mounting fluid under the cover slip.

-prevent seepage of the mountant.

-immobilize cover slip

- prevent sticking of slides on storage.

- Decoration, different colors can be applied.

Application of molten paraffin wax or nail vanish is achieved by use of a straight metal rod or a brush around the rectangular cover slip.

ROUTINE BASIC STEPS IN STAINING AND MOUNTING OF PARAFFIN WAX SECTIONS

- Removal of paraffin wax using xylene.

- Hydrating with descending grades of alcohol

- Staining

- Dehydrating using ascending grades of alcohol

- Clearing in xylene

- Mounting in D.P.X under a cover slip.